Project Roadmap: Replicating Ting-Yu Chen's Portfolio

Open-source equivalents  |  6 projects  |  ~24 weeks  |  No commercial EDA license needed

# 1. Context & Honest Assessment

Ting-Yu Chen is a UIUC ECE grad student currently working as a DFT Engineer at Astera Labs. Her 7 projects span custom IC layout, RTL design, FPGA implementation, and SoC integration — a genuinely strong portfolio. The goal of this roadmap is to build equivalent open-source versions of 6 of her 7 projects (skipping the wet-lab electroless plating research which requires physical lab access). Each project below maps her version to a free-tool equivalent that produces comparable learning and demonstrable GitHub output.

⚠  Projects 1, 2, 4 in her list use Cadence Virtuoso and Synopsys DC — commercial tools costing lakhs/year in license fees, available only through universities. The equivalents below use OpenROAD, Yosys, ngspice, and Xilinx Vivado (free) and are fully demonstrable without any license.

# 2. Prerequisites — Learn These First (Weeks 1–6)

Do NOT start any project without these. Jumping to P1 without Verilog basics = wasted time and frustration.

| Prerequisite | Learn by | Priority | Why you need it |
| --- | --- | --- | --- |
| Digital logic fundamentals | Week 1 | 🔴 Must | Boolean algebra, combinational circuits, flip-flops, MUX, XOR, shift registers |
| Verilog basics | Week 1–2 | 🔴 Must | Module, always block, wire/reg, structural vs behavioral — needed for every project |
| SystemVerilog (SV) | Week 2–3 | 🔴 Must | Interfaces, enums, logic type, SV-specific constructs — used in P3 through P6 |
| FSM design in HDL | Week 2 | 🔴 Must | Moore/Mealy machines in Verilog/SV — core of LC-3 control unit (P3) |
| RISC-V ISA basics | Week 3 | 🔴 Must | RV32I instruction set — needed to understand P2, P4 datapath |
| CMOS logic design | Week 3–4 | 🟡 Important | Transistor-level gates, transmission gate logic — needed for P2 custom cell library |
| FPGA toolchain (Vivado) | Week 3 | 🔴 Must | Project setup, synthesis, implementation, bitstream — needed for P3/P4/P5/P6 |
| AXI protocol (AXI4-Lite) | Week 5 | 🔴 Must | Register map, read/write transactions — needed for P4, P5, P6 |
| AXI4-Stream + DMA | Week 6 | 🔴 Must | Stream handshaking, scatter-gather DMA — needed for P6 (hardest project) |
| Bare-metal C on ARM | Week 4 | 🔴 Must | Pointers, memory-mapped I/O, UART — needed for P6 Zynq firmware |
| Tcl scripting | Week 2 | 🟡 Important | You already have Python — Tcl is similar. Used in P4 (Synopsys DC / Innovus) |
| OpenROAD / Yosys flow | Week 4 | 🟡 Important | Open-source alternative to Synopsys DC + Innovus for P4 equivalent |
| Timing concepts (setup/hold) | Week 3 | 🔴 Must | WNS, TNS, clock uncertainty — needed to interpret P2/P4 results |
| HSPICE / ngspice basics | Week 5 | 🟢 Helpful | Circuit simulation for P1 custom analog cells — ngspice is free alternative |

## Key Free Resources for Prerequisites

- Verilog/SV: HDLBits — interactive exercises

- SystemVerilog: ChipVerify SV Tutorial

- RISC-V ISA: RISC-V Reader (free PDF)

- FPGA / Vivado: Nandland FPGA Tutorials

- AXI Protocol: Xilinx AXI Reference Guide (PG344)

- CMOS logic: Weste & Harris — CMOS VLSI Design (free chapters)

- OpenROAD: OpenROAD Docs

- Tcl: Tcl Tutorial (Tcler's Wiki)

# 3. Master Timeline — 24 Weeks

| Period | Phase | Focus | Activities | Milestone |
| --- | --- | --- | --- | --- |
| Wk 1–2 | Prerequisites | Digital logic, Verilog basics, FSM | Verilog tutorial + HDLBits exercises | — |
| Wk 3–4 | Prerequisites | SystemVerilog, RISC-V ISA, Vivado setup | SV Intro + Vivado install + RV32I reading | — |
| Wk 5–6 | Prerequisites | AXI4-Lite, CMOS logic, Tcl, timing concepts | AXI spec reading + CMOS textbook ch 1–3 | — |
| Wk 7–8 | P1 | Face Detector — XOR comparator + PISO shift reg | Design → simulate in Icarus/ModelSim → FPGA | GitHub: P1 ✓ |
| Wk 9–10 | P2 start | RISC-V datapath: ALU + register file + custom cells | Bitslice ALU in SV, simulate all ops | GitHub: P2a ✓ |
| Wk 11–12 | P2 finish | Integrate datapath, OpenROAD layout | Full datapath PnR in OpenROAD, area report | GitHub: P2b ✓ |
| Wk 13–14 | P3 | Automated PD — Yosys synth + OpenROAD PnR + Tcl | Write Tcl flow scripts, run GDSII gen | GitHub: P3 ✓ |
| Wk 15–16 | P4 | LC-3 multi-cycle processor in SV on FPGA | FSM controller + datapath + Vivado impl | GitHub: P4 ✓ |
| Wk 17–18 | P5 | HDMI Text Controller IP + AXI4-Lite | VGA/HDMI timing, char ROM, AXI slave | GitHub: P5 ✓ |
| Wk 19–20 | P6 | AXI Caesar Cipher on Zynq + bare-metal C | PL core in SV, PS firmware in C, DMA | GitHub: P6 ✓ |
| Wk 21–24 | Job Prep | Resume, LinkedIn posts, mock interviews | Per-project write-up + application blast | Offer ready |

# 4. Project-by-Project Breakdown

Each project card shows: her version vs. your open-source version, prerequisites needed, and what to deliver on GitHub.

| P1 Face Detector System — XOR Pixel Comparator + PISO Shift Register |  |  |
| --- | --- | --- |
|  | Her Version (UIUC) | Your Version (Open-Source) |
| Tools | Cadence Virtuoso + HSPICE, 45nm GPDK, 2.4 GHz | Verilog + Icarus iverilog + GTKWave + Vivado (FPGA impl) |
| Duration | Her: ~2 months (Oct–Dec 2024) | You: ~2 weeks (Wk 7–8) |
| Difficulty | Advanced (analog timing closure) | Intermediate (digital logic, well-documented) |
| What it is | Compares two 4×4 grayscale images pixel by pixel using XOR logic. Counts differing pixels. Uses a PISO (Parallel-In Serial-Out) shift register to serialize results for transmission. Her version was implemented at transistor level in 45nm. Your version is RTL in Verilog, simulated and then implemented on FPGA. |  |
| Your prerequisites | ✅ Digital logic (XOR, MUX, flip-flops) ¦ ✅ Verilog module writing ¦ ✅ Testbench basics ¦ ✅ Vivado or iverilog installed |  |
| Key deliverables | • 4×4 XOR comparator module in Verilog with full testbench • PISO shift register module with simulation waveforms (GTKWave screenshot) • Top-level integration + FPGA bitstream (Basys3 or Nexys) • README with block diagram (draw it in draw.io), timing analysis • GitHub repo: pixel-comparator-fpga |  |
| Learn from | HDLBits — Shift Registers exercises |  |

| P2 Custom RISC-V 32-bit Datapath Layout — Bitslice + Custom Cell Library |  |  |
| --- | --- | --- |
|  | Her Version (UIUC) | Your Version (Open-Source) |
| Tools | Cadence Virtuoso, 45nm GPDK, custom CMOS cells, transmission gate logic | Yosys + OpenROAD + Sky130 PDK + KLayout (GDSII viewer) |
| Duration | Her: ~2 months (Feb–Apr 2025) | You: ~4 weeks (Wk 9–12) |
| Difficulty | Most complex — requires CMOS + layout knowledge | Hard but achievable with OpenROAD automation |
| What it is | Designs a 32-bit RISC-V processor datapath using a bitslice (column-slice) approach. Builds a custom standard cell library implementing gates in CMOS with transmission-gate XOR/XNOR/MUX for area efficiency. Final layout integrates 32 vertically stacked bitslices. Your version uses Sky130 open PDK and OpenROAD for automated placement and routing, skipping transistor-level custom layout (which requires Virtuoso). |  |
| Your prerequisites | ✅ Verilog/SV for ALU, register file, mux ¦ ✅ RISC-V RV32I ISA ¦ ✅ Basic CMOS logic understanding ¦ ✅ Yosys + OpenROAD installed ¦ ✅ Timing concepts (setup/hold) |  |
| Key deliverables | • RV32I ALU in SystemVerilog: ADD, SUB, AND, OR, XOR, SLT, shifts — all ops with testbench • Register file (32×32-bit) in SV with simulation • Full datapath integration in SV (ALU + reg file + mux + forwarding) • Yosys synthesis script → netlist (with cell count, area stats) • OpenROAD PnR → GDSII → KLayout screenshot • README: block diagram, cell count table, timing report snippet • GitHub repo: riscv32-datapath-openroad |  |
| Learn from | RISC-V CPU Design Guide (Cornell ECE4750 public labs) |  |

| P3 Automated Physical Design of RISC-V Processor — RTL to GDSII |  |  |
| --- | --- | --- |
|  | Her Version (UIUC) | Your Version (Open-Source) |
| Tools | Synopsys Design Compiler (synthesis) + Cadence Innovus (PnR) + Tcl | Yosys (synthesis) + OpenROAD (PnR) + Tcl scripting + GitHub Actions CI |
| Duration | Her: ~2 months (Apr–May 2025) | You: ~2 weeks (Wk 13–14) — builds on P2 |
| Difficulty | Advanced (full flow automation) | Intermediate — you already have OpenROAD from P2 |
| What it is | Automates the complete RTL-to-GDSII physical design flow of a RISC-V processor. Uses Tcl scripting to drive synthesis → floorplan → placement → CTS → routing → DRC signoff. Produces a GDSII-ready layout. Her version uses Synopsys DC + Innovus (industry tools). Your version uses Yosys + OpenROAD which produce equivalent results and are fully open-source — arguably more impressive because you built the flow from scratch. |  |
| Your prerequisites | ✅ Tcl scripting basics ¦ ✅ OpenROAD working from P2 ¦ ✅ P2 RTL (reuse same design) ¦ ✅ Timing concepts ¦ ✅ Bash scripting (you already did this) |  |
| Key deliverables | • Full Tcl automation script: synth.tcl + floorplan.tcl + pnr.tcl + signoff.tcl • Python QoR report parser (reuse + extend your eda-log-analyzer from Day 1) • GitHub Actions CI: auto-run flow on every push, artifact the reports • KLayout GDSII screenshot of final routed layout • WNS/TNS/area/utilization table in README • GitHub repo: riscv-pd-automation (this is the star project — most employer-visible) |  |
| Learn from | OpenROAD Flow Scripts (full example flows) |  |

| P4 LC-3 Multi-Cycle Processor — SystemVerilog + FPGA |  |  |
| --- | --- | --- |
|  | Her Version (UIUC) | Your Version (Open-Source) |
| Tools | SystemVerilog + Xilinx Vivado + FPGA board | SystemVerilog + Xilinx Vivado (free) + Basys3/Nexys FPGA |
| Duration | Her: ~2 months (Jun–Jul 2025) | You: ~2 weeks (Wk 15–16) |
| Difficulty | Advanced (processor design + FSM) | Same difficulty — tools are identical (Vivado is free) |
| What it is | Designs and implements a complete LC-3 multi-cycle processor in SystemVerilog on FPGA. Includes both the datapath (ALU, registers, memory) and an FSM-based control unit that supports the full LC-3 instruction set: ALU ops, load/store, branches, subroutine calls (JSR/JMP), and self-modifying code. This is one of the closest 1:1 replications because Vivado is free and you use the same tools she did. |  |
| Your prerequisites | ✅ SystemVerilog (interfaces, enums, logic) ¦ ✅ FSM design ¦ ✅ Vivado installed ¦ ✅ LC-3 ISA (read the LC-3 ISA manual — free PDF) |  |
| Key deliverables | • FSM controller module in SV: all 16 LC-3 states, correct transitions • Datapath module: ALU, register file, PC, MAR, MDR, IR • Top-level integration + Vivado constraint file (.xdc) • Testbench running 5+ LC-3 programs: ADD, AND, LD, ST, BR, JSR • FPGA demo: show LED/7-seg output of running a simple program • README with FSM state diagram (draw.io), waveform screenshots • GitHub repo: lc3-multicycle-sv-fpga |  |
| Learn from | LC-3 ISA Reference Manual (free PDF) |  |

| P5 HDMI Text Mode Controller IP — AXI4-Lite SoC Integration |  |  |
| --- | --- | --- |
|  | Her Version (UIUC) | Your Version (Open-Source) |
| Tools | SystemVerilog + Vivado + FPGA + AXI4-Lite slave | SystemVerilog + Vivado + Basys3/Nexys4 + VGA (or HDMI if board has it) |
| Duration | Her: ~2 months (Jul–Aug 2025) | You: ~2 weeks (Wk 17–18) |
| Difficulty | Advanced (video timing + AXI) | Hard but well-documented — VGA timing is straightforward |
| What it is | Implements a custom HDMI/VGA text mode controller IP that renders an 80×30 character display (640×480 pixels) on a monitor. The controller is integrated with an AXI4-Lite slave interface, enabling it to be used as an SoC peripheral. Characters are stored in a memory-mapped register map that the CPU (or AXI master) writes to. Your version targets VGA (most FPGA dev boards have VGA) — if your board has HDMI, use that. |  |
| Your prerequisites | ✅ SystemVerilog ¦ ✅ AXI4-Lite protocol (read/write transactions, register map) ¦ ✅ Vivado with IP integrator ¦ ✅ Basic VGA/HDMI timing (640×480 @ 60Hz pixel clock) |  |
| Key deliverables | • VGA timing controller in SV: hsync, vsync, pixel clock (25MHz for 640×480) • Character ROM: 8×8 font stored in BROM, indexed by ASCII code • Text framebuffer: 80×30 array of 7-bit ASCII registers • AXI4-Lite slave: CPU writes char code to address, controller reads and renders • Integration in Vivado block design with MicroBlaze or Zynq PS • Demo: display your name + project title on monitor via FPGA • GitHub repo: axi-vga-text-controller-ip |  |
| Learn from | FPGA VGA Controller Tutorial (Nandland) |  |

| P6 AXI Caesar Cipher Accelerator — Zynq SoC + Bare-Metal C Firmware |  |  |
| --- | --- | --- |
|  | Her Version (UIUC) | Your Version (Open-Source) |
| Tools | SystemVerilog + Vivado + Xilinx Zynq SoC + C + AXI4-Stream + DMA | SystemVerilog + Vivado + Zynq (Pynq-Z2/Zybo) or Nexys + MicroBlaze + C |
| Duration | Her: ~2 months (Sep–Oct 2025) | You: ~2 weeks (Wk 19–20) |
| Difficulty | Most complex — Zynq PS+PL co-design | Most complex — same tools, same difficulty |
| What it is | Implements a Caesar cipher hardware accelerator on a Zynq SoC. The PL (FPGA fabric) contains the cipher core in SystemVerilog, exposed via AXI4-Lite (for shift amount register) and AXI4-Stream TX/RX (for data). The PS (ARM CPU) runs bare-metal C firmware that configures the shift, sets up DMA scatter-gather descriptors, and streams text through the accelerator via UART. Demonstrates full HW/SW co-design. If you don't have a Zynq board, use MicroBlaze softcore on Nexys4 as the PS equivalent. |  |
| Your prerequisites | ✅ SystemVerilog + AXI4-Lite (from P5) ¦ ✅ AXI4-Stream protocol ¦ ✅ Bare-metal C: pointers, memory-mapped I/O, UART ¦ ✅ DMA concepts ¦ ✅ Vivado + Vitis/SDK |  |
| Key deliverables | • Caesar cipher core in SV: configurable shift, stream in/out • AXI4-Lite slave: single register for shift amount • AXI4-Stream master/slave interfaces for data TX/RX • Bare-metal C firmware (Vitis): UART input → DMA → accelerator → UART output • Demo: type text on PC terminal → FPGA encrypts → prints ciphertext back • README: block diagram, AXI register map table, demo video link (loom.com) • GitHub repo: zynq-caesar-cipher-axi |  |
| Learn from | Zynq Book (free PDF) — PS+PL co-design guide |  |

# 5. Tool Map — Her Tools vs. Your Free Equivalents

| Her Tool (Commercial) | Your Equivalent (Free) | Notes |
| --- | --- | --- |
| Cadence Virtuoso (schematic + layout) | KLayout + OpenROAD + Magic VLSI | Layout viewing, DRC — Sky130 PDK supported |
| HSPICE (circuit simulation) | ngspice (free, SPICE-compatible) | Transient, DC, AC simulation — same netlist format |
| Synopsys Design Compiler (synthesis) | Yosys (open-source synthesis) | RTL → gate-level netlist, timing-driven synthesis |
| Cadence Innovus (place & route) | OpenROAD (open-source PnR) | Floorplan, placement, CTS, routing, GDSII export |
| Xilinx Vivado (FPGA) | Xilinx Vivado (free WebPACK edition) | EXACT same tool — no difference |
| Synopsys VCS (simulation) | Icarus iverilog + GTKWave (free) | Verilog/SV simulation + waveform viewing |
| Cadence Genus (synthesis alt) | Yosys + ABC | Technology mapping, optimization |
| ModelSim/QuestaSim | Icarus iverilog or Verilator | Fast functional simulation |

# 6. GitHub Strategy — How to Present These Projects

## Repo naming convention

- Use descriptive, tool-tagged names: riscv32-datapath-openroad, lc3-multicycle-sv-fpga, zynq-caesar-cipher-axi

- Every repo needs: README.md, block diagram image, simulation waveform screenshot, results table

- Pin your 3 best repos on your GitHub profile page

## README structure for every project (copy this format)

- 1 paragraph: what the project does and why it matters

- Block diagram image (draw.io — free at app.diagrams.net)

- Tools used table: tool name + version + purpose

- Key results table: metrics like cell count, WNS, area, utilization, fault coverage

- How to run: 3–5 step reproduction instructions

- Waveform / layout screenshot — visual proof it runs

## Commit discipline

- Commit after every working module — not after the whole project

- Commit messages: 'feat: add ALU with SUB and AND ops' not 'update code'

- Use branches: main (stable) + dev (working)

# 7. Honest Notes

Her profile is strong because of two things: the breadth (custom layout + RTL + FPGA + SoC) and the depth within each project (process corners, GDSII, AXI DMA). Your open-source versions will match the breadth. The depth will be different in kind — no 45nm tape-out — but you will have something she doesn't: a fully reproducible, public, open-source flow that any engineer in the world can clone and run. That is genuinely valuable and increasingly respected in the Indian semiconductor hiring market.

- P3 (Automated PD with Tcl + OpenROAD + CI) is your strongest project — it directly maps to EDA automation roles and DFT flow engineer roles in India

- P6 (Zynq Caesar Cipher) is your most impressive to embedded hiring managers — it shows HW/SW co-design end to end

- P4 (LC-3 FPGA) is the most teachable — you will understand processor architecture deeply after this, which helps in DFT interviews

- Do them in order — each project reuses skills and sometimes code from the previous one

Generated March 2026  |  Based on Ting-Yu Chen's LinkedIn profile  |  Open-source equivalents roadmap